🔁 Network Address Translation (NAT) – Complete Guide

1️⃣ What is NAT?

NAT (Network Address Translation) converts private IP addresses into public IP addresses (and vice versa) so devices can communicate across different networks—mainly the Internet.

Why NAT is used

- Conserves IPv4 addresses

- Adds a basic layer of security

- Allows private networks to access public networks

- Enables multiple devices to share one public IP

2️⃣ Types of IP Addresses in NAT

| Term | Meaning |
| --- | --- |
| Inside Local- | Private IP of inside device (e.g. 192.168.1.10) |
| Inside Global- | Public IP representing inside device (e.g. 200.65.110.0) |
| Outside Local- | IP of external device as seen internally |
| Outside Global- | Real public IP of external device |

3️⃣ Types of NAT

3.1 Static NAT (1-to-1)

Definition

Maps one private IP to one public IP permanently.

Use cases

Web servers

Mail servers

FTP servers

DNS servers

Characteristics

✔ Permanent mapping
✔ External devices can initiate connections
❌ Wastes public IPs

Example

192.168.1.10 ↔ 41.90.10.5

3.2 Dynamic NAT (Many-to-Many)

Definition

Maps private IPs to a pool of public IPs dynamically.

Characteristics

✔ Saves IPs compared to static NAT
❌ No guarantee of same public IP
❌ Outside cannot initiate connections

Example

## Public pool: 41.90.10.1 – 41.90.10.10

3.3 PAT (Port Address Translation) – NAT Overload

Maps many private IPs to one public IP using port numbers.

Characteristics

✔ Most common NAT
✔ Used in home & enterprise networks
✔ Conserves IPv4 addresses

Example

192.168.1.10:1025 → 41.90.10.1:30001

192.168.1.11:1026 → 41.90.10.1:30002

6️⃣ Port Forwarding (Static PAT)

Maps a public IP + port to a private IP + port.

Example

41.90.10.5:80 → 192.168.1.10:80

Use cases

Web servers

CCTV access

Remote desktop

7️⃣ NAT Order of Operation (IMPORTANT 🔥)

Routing decision

NAT translation

Packet forwarding

Many engineers miss this in troubleshooting.

NAT Terminology Summary

| Static NAT |  | One-to-one |
| --- | --- | --- |
| Dynamic NAT |  | Many-to-many |
| PAT |  | Many-to-one |
| Inside |  | Your private network |
| Outside |  | External network |

NAT Troubleshooting Checklist

✔ Check inside/outside interfaces
✔ Verify ACL used in NAT
✔ Check NAT translations
✔ Verify routing
✔ Ensure correct NAT type

Commands:

show ip nat translations

show ip nat statistics